## Experiment–1

Title: Study of Different Types of Network Cables and Practical Implementation of Cross-wired and Straight-through Cable using Clamping Tool

### Aim

To study various network cables and practically create a cross-wired cable and a straight-through cable using a twisted pair cable, RJ-45 connectors, and a crimping tool.

### Apparatus / Components Required

- RJ-45 Connectors (2 for each cable)

- Crimping Tool

- Cable Stripper / Cutter

- Twisted Pair Cable (Cat5e / Cat6)

- Network Cable Tester

### Theory

Network cables are essential for wired communication in computer networks. Twisted Pair Cables are the most commonly used in LANs, especially Category 5e (Cat5e) and Category 6 (Cat6) cables.
They can be terminated in two wiring standards: T568A and T568B.

1. Straight-through Cable

- Both ends follow the same wiring standard (T568A–T568A or T568B–T568B).

- Used to connect different devices (e.g., PC to Switch, Switch to Router).

2. Cross-over Cable

- One end follows T568A, the other end follows T568B.

- Used to connect similar devices (e.g., PC to PC, Switch to Switch).

Colour Codes:

- T568A:

  - White-Green

  - Green

  - White-Orange

  - Blue

  - White-Blue

  - Orange

  - White-Brown

  - Brown

- T568B:

  - White-Orange

  - Orange

  - White-Green

  - Blue

  - White-Blue

  - Green

  - White-Brown

  - Brown

### Procedure

- Strip Cable Jacket – Using a cable stripper, remove about 2 inches of the outer jacket without damaging the internal wires.

- Separate and Arrange Wires – Untwist and align wires according to the T568A or T568B standard.

- Prepare Cable Ends –

  - For Straight-through: Arrange both ends in the same standard.

  - For Cross-over: Arrange one end as T568A and the other as T568B.

- Trim Wires – Cut all eight wires evenly to about 0.5 inch from the jacket.

- Insert into RJ-45 Connector – Push wires into the connector ensuring they are in the correct slots and fully seated.

- Crimp Connector – Place the connector into the crimping tool and press firmly to secure.

- Test the Cable – Use a network tester to verify correct pin connections.

### Result/Conclusion

We have successfully studied and implemented straight-through and cross-over network cables using twisted pair cable, RJ-45 connectors, and a crimping tool, and tested their working using a cable tester.

### Viva Questions

- What is the difference between straight-through and cross-over cables?

- What are the wiring color codes for T568A and T568B standards?

- Which cable is used to connect two PCs directly?

- What is the function of the RJ-45 connector?

- Why should we minimize the untwisting of twisted pair cables?

- What is the maximum cable length allowed for Ethernet using twisted pair cable?

- Why do we use a network tester after making cables?

- Which category of cable supports Gigabit Ethernet?