## Experiment–4

Title: Creating a Simple LAN in Cisco Packet Tracer and Testing Connectivity using the Ping Command

### Aim

To design and implement a basic Local Area Network (LAN) in Cisco Packet Tracer using PCs and a switch, assign IP addresses, and test communication using the ping command.

### Apparatus / Software Required

- Cisco Packet Tracer (installed)

- Computer/Laptop

- Basic knowledge of IP addressing

### Theory

A Local Area Network (LAN) connects devices within a limited area such as a classroom, office, or home.

Key Components in this Experiment:

- PCs (End Devices) – Represent network hosts.

- Switch – Connects multiple devices in the same LAN and forwards frames using MAC addresses.

- Copper Straight-Through Cable – Used to connect PCs to the switch.

- IP Address – A unique identifier for each device in the network.

Ping Command:

- Used to test network connectivity by sending ICMP Echo Request messages to another device.

- If the destination is reachable, it replies with an ICMP Echo Reply.

### Procedure

- Open Packet Tracer and select Logical Workspace.

- Add Devices:

  - Drag 3 PCs from End Devices section into the workspace.

  - Drag 1 Switch (2960 model) from Switches section into the workspace.

- Connect Devices:

  - Use the Copper Straight-Through Cable to connect each PC to the switch.

- Assign IP Addresses to PCs:

  - Click on PC0 → Desktop tab → IP Configuration → Enter:

    - IP Address: 192.168.1.1

    - Subnet Mask: 255.255.255.0

  - Repeat for PC1 and PC2 with IPs:

    - PC1: 192.168.1.2

    - PC2: 192.168.1.3

- Test Connectivity:

  - On PC0, go to Desktop → Command Prompt.

  - Type: ping 192.168.1.2 → Press Enter.

  - Observe the Reply messages indicating successful communication.

- Repeat Ping between all PCs to confirm full connectivity.

| Source PC | Destination PC | Command Used | Result |
| --- | --- | --- | --- |
| PC0 | PC1 | ping 192.168.1.2 | Success / Fail |
| PC0 | PC2 | ping 192.168.1.3 | Success / Fail |
| PC1 | PC2 | ping 192.168.1.3 | Success / Fail |

### Result

A simple LAN was successfully created in Cisco Packet Tracer by connecting PCs via a switch, assigning IP addresses, and testing connectivity with the ping command.

### Viva Questions

- What is the purpose of a switch in a LAN?

- Why do we use copper straight-through cables in this setup?

- What is the difference between a switch and a hub?

- What does a successful ping indicate?

- What will happen if two devices in a LAN have the same IP address?

- What is the default subnet mask for a Class C IP address?

- Can ping work without IP configuration? Why or why not?